# Lenses For Various Formats

## N. David King

Most beginning photographers gain familiarity with the way the lenses work for their first camera formats.  If, for example, you start out with a 35mm format you quickly learn that a “Normal” lens has a focal length of about 50mm (45-60mm) and soon acquire a sense of what that lens “sees,” i.e. what its field of view covers.  But when adding a new format to their equipment list it can be confusing since the lens that sees about the same field of view for a square medium format is 75-80mm and for a 4x5 view camera is about 127-135mm.

Below is a table that gives the approximate equivalent lens focal lengths that yield the same field of view in different formats.  Remember however, for issues such as Depth of Field, even though the field of view varies between formats, the depth of field is the same from a 100mm for a small format as it is for a 100mm for a large format.

All lens focal lengths are shown in millimeters for comparison even though often large format lenses are listed in inches.

| Format | Wide Angle |  |  |  |  | Normal | Telephoto |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35mm | 10 | 15 | 20 | 24 | 28 | 50 | 85 | 100 | 135 | 150 | 200 | 300 |
| 6.45x6 | N/A | 20 | 30 | 35 | 40 | 70-75 | 135 | 150 | 200 | 250 | 300 | 450 |
| 6x6 |  | 26 | 35 | 45 | 50 | 75-80 | 150 | 175 | 250 | 275 | 375 | 525 |
| 6x7 |  | 30 | 40 | 43 | 55 | 90 | 170 | 200 | 275 | 300 | 400 | 600 |
| 4x5 |  | 40 | 50 | 75 | 90 | 127-135 | 270 | 320 | 450 | 570 | 660 | 980 |
| 8x10 |  | 95 | 135 | 160 | 196 | 345 | 600 | 680 | 930 | 1045 | 1375 | 2085 |
| View |  | 110 | 90 | 80 | 70 | 45 | 30 | 25 | 17 | 15 | 12 | 8 |